Water layer on mica in ambient conditions

(from Israelachvili chapter 11)

Hamaker constant, A, mica (1) – water (3) – air (2) :

11.13:Atotal≈34kTε1−ε2ε1+ε3ε2−ε3ε2+ε3+3hνe82n12−n32n22−n32n12+n32n22+n32n12+n32+n22+n32

|  | Water | Mica | Air |
| --- | --- | --- | --- |
| ε (dielectric constant) | 80 | 7 | 1 |
| n (refractive index) | 1.33 | 1.6 | 1 |

νe= 3 1015 s-1, kT= 27.6 10-23J, h=6.6 10-34 Js

→  Awater−mica−air= -2.4 10-20J

Thickness of the water layer, D:

Example p. 195:

pp0=expAυ6πD3kT   →   D=3Aυ6πkTlogpp0 ,

υ: molecular volume of water = 29.9 10-30m3

| Humidity [%] | Water layer thickness, D , [nm] |
| --- | --- |
| 50 | 0.77 |
| 70 | 0.96 |
| 90 | 1.44 |
| 99 | 3.16 |
| X | 3−0.138log(X100) |