----------------------------------------------------------------------

Date:    Tue, 20 Feb 2001 14:40:03 -0700

From:    Rick Workman <rworkman@U.Arizona.EDU>

Subject: re: cleaning calibration standard grid

Message-ID: <#1>

We have found that casting a drop of Dow Corning Sylgard 184 (mixed

according to directions) onto our grating, curing, then peeling off the

cured silicone cleans the grating very thoroughly.  We've done this to all

of the 3 micron pitch gratings in the NT-MDT grating set TGS02 without any

damage to the gratings.  As a bonus, you get a nice replica of your grating.

This is the same stuff used by Whitesides' group and many others for

microcontact printing.

We ordered our's from

K.R. Anderson

2800 Bowers Ave

Santa Clara, CA

1-408-727-2800

I think their minimum order is $50, but the Sylgard costs only 40 something

for the 1lb size, so you may have to eat the difference.  I've recieved

terrible service from this company in past (nearly 6 months to receive my

second order), so beware.

Rick Workman

University of Arizona

Dept. of Materials Science

Tucson, AZ 85721

------------------------------

Date:    Tue, 20 Feb 2001 17:00:59 EST

From:    Co2clean <Co2clean@aol.com>

Subject: Re: Cleaning Calibration Items

Message-ID: <#2>

CO2 Snow Cleaning has been used to by many AFM and SPM users to clean samples

and other items related to Force Microscopy.  Please see www.co2clean.com and

the AFM examples at www.co2clean.com/afm.htm.

Robert Sherman

Applied Surface Technologies

roberts@co2clean.com

908-464-6675

------------------------------

Date:    Tue, 20 Feb 2001 15:43:06 -0800

From:    Tim Senden <tjs110@rsphysse.anu.edu.au>

Subject: cleaning calibration standard grid

Message-ID: <#3>

My 2 cents worth is to try carbon dioxide snow cleaning.  It is without a

doubt the best way of removing particulates.  We have cleaned old grids

with the technique with no damage to the gold ruling.  It will easily

remove finger prints from a gold surface.  We built our own, but I'd

recommend you just go straight to the company that makes them and buy one.

See http://www.co2clean.com for further info.  One other thing is to have a

nitrogen gun on hand to blow off any condensate (water ice).  I'd recommend

boil-off from liq. N2 rather than bottled gas as it is much, much cleaner.

Tim.

(PS. I have no commercial connection with the company mentioned above.)

*****************************************************

Dr Tim Senden

Dept. Applied Mathematics

Research School of Physical Sciences and Engineering

The Australian National University

Canberra   ACT   0200  AUSTRALIA

Ph. +61-2-61 25 47 81 or +61-2-61 25 33 26

Fax:+61-2-61 25 07 32

http://rsphysse.anu.edu.au/appmaths

*****************************************************

------------------------------