IMAGING OF PURPLE MEMBRANE BY AFM

Sample preparation:

- Take purple membrane from –80 freezer (aliquot is 10 l, conc. 6 g/l )

- Dilute it with 250 l of buffer A

- Put a 75 l droplet of diluted purple membrane on a freshly cleaved mica surface with a silicone border

- Store this sample at 4 (fridge) during 20 minutes

- Rinse the sample with buffer A (6x10 times), prevent drying of the sample!

- Image in buffer A in contact mode with the softest cantilever (k=0.06 N/m)

Buffer A: pH=7.6, 300 mM KCl, 10 mM Tris

We used the e-scanner; imaging was in contact mode in liquid, with a tip

velocity of 5 micron/s. The tip was the k=0.06 N/m cantilever from a

standard DI contact mode chip. The force was kept as low as possible by

adjusting the setpoint manually during scanning.

Sample preparation:

Store the purple membrane at -80 degrees C. Dilute it to 240 microg/ml

for the sample preparation. Incubate a droplett of 75 microl. during 20

minutes at 4 degrees C on mica; then rinse it very well with buffer. Now

the sample is ready for AFM imaging. The buffer solution for adsorption

and imaging was the same: pH 7.6 300mM KCl 10 mM Tris.

What should be tested:

- Imaging in 150 mM?

- Stirring the purple membrane solution for 6 seconds (Linz), maybe in this way we can get rid of the white blobs on the membrane surfaces

- In Linz they use pH 8.8, 150 mM KCl, 10 mM Tris

- The influence of the imaging parameters: scan speed and applied force

LINZ EXPERIMENTAL

To clean the fluid cell, tip holder and tip:

- SDS 10% (detergent)

- ethanol

- DI water

- dry with tissue (and blow dry)

Filter the buffer (0.3 m pores)

Purple membrane can be stored for a few days at –20 and reused 3 times. We have seen that the membrane deteriorates if it is not kept at –80!